AHB VIP Verification Plan

- 1. Introduction

- AMBA AHB (Advanced High-performance Bus) is an on-chip communication protocol introduced by ARM as part of the Advanced Micro-controller Bus Architecture (AMBA) specification. It is designed for high-speed, high-bandwidth, and high-clock-frequency system modules in System-on-Chip (SoC) designs. An AHB verification plan defines the features to verify, stimulus to generate, assertions to implement, and functional coverage required to demonstrate that the design complies with the protocol specification.

2. Verification Objective

- Verify correct AHB transfer behavior for all supported transfer types.

- Verify address and data phase timing.

- Verify single and burst transfers.

- Verify HREADY wait-state handling and HRESP response behavior.

- Verify correct handling of HTRANS, HBURST, HSIZE, and HWRITE.

- Verify back-to-back transfers.

- Achieve functional and code coverage closure.

- Check protocol compliance using assertions and a reference model.

- 3. Verification Testbench

- Testbench Methodology: UVM based

- Architecture:

- Testbench Top:

- Clock and reset generation - HCLK and HRESETn

- Interface instantiation - ahb_if

- config db registration - uvm_config_db

- starting simulation - run_test

- Test: Builds environment, start the sequences and manage simulation objections.

- Sequences: It creates and randomizes (stimulus generation) the data inside sequence item and send to driver via sequencer. Sequences(different scenarios) are inherited from base sequence.

- Environment: Encapsulates Master agent, slave agent, scoreboard and coverage.

- AHB Master Agent: It encapsulates sequencer, driver and monitor.

- Driver drives the stimulus to interface like address, control signals and write data.

- Monitor samples the signal from interface and broadcasts completed transaction via uvm_analysis_port.

- Sequencer routes generated sequence item to the driver.

- AHB Slave Agent:

- Driver responds to transactions and drives the read data, ready and response signals and memory is also initialized in this driver.

- Monitor samples the signal from interface and broadcasts completed transaction via uvm_analysis_port.

- Sequencer routes generated sequence item to the driver.

- Scoreboard: Compares expected vs actual results. It also includes the reference memory which act as a expected results.

- Coverage: Coverage sampling, bins, and cross coverage.

Interface: Act as a bridge between Master and Slave.

- Assertions: Cycle-level protocol rule checking.

4. Verification Strategy

The AHB VIP will be verified using a UVM-based testbench consisting of master and slave agents, monitors, a scoreboard, functional coverage in the environment, and protocol assertions in the interface.

- Test Plan:

- The test plan defines the test scenarios used to verify AHB VIP functionality and protocol behavior. Test_Plan

| Test scenario | Verification objective |
| --- | --- |
| Single read test | Verify read transaction generation and monitoring |
| Single write test | Verify write transaction generation and monitoring |
| Back-to-back transfers | Verify pipelined transaction handling |
| Transfer size test | Verify all supported transfer sizes |
| Burst transfer test | Verify supported burst types and beat sequencing |

- Coverage Plan:

- The coverage plan defines the functional coverage points and combinations needed to demonstrate that the required AHB features have been exercised. Coverage is implemented in the UVM environment.

| Coverage feature | Coverage goal |
| --- | --- |
| Transfer type | Cover all applicable transfer types |
| Transfer direction (HWRITE) | Cover reads and writes |
| Transfer size (HSIZE) | Cover all supported sizes |
| Burst type (HBURST) | Cover all supported burst types |
| Wait states (HREADY) | Cover zero and multiple wait states |
| Response (HRESP) | Cover applicable successful and error responses |
| Address regions | Cover configured address ranges and boundaries |
| Direction × size | Cover required read/write and size combinations |
| Burst × size | Cover legal burst and size combinations |

5. Simulation

| Simulation option | Description | Expected result |
| --- | --- | --- |
| Compilation | Compile the AHB VIP UVM testbench using Makefile | Compilation completes without errors |
| Test selection (sequence selection) | Select a sequence using a Makefile variable | Selected UVM sequence executes |
| Random seed | Pass random seed through Makefile | Simulation runs with the specified seed |
| Assertion control | Enable/disable assertions through simulation options | Assertions behave according to configuration |
| Coverage control | Enable functional coverage collection | Coverage data is generated |
| Waveform generation | Enable waveform dumping through Makefile | Waveform file is generated |
| Regression | Execute multiple sequences using Makefile | All selected sequences execute and results are reported |
| Clean | Clean generated simulation files | Temporary and generated files are removed |

- Run the code: Make command Makefile

- Example: make sim seed=100 child=ahb_burst WAVE=1

- sim target will first run the target compile then run the simulation command. Seed will 100 and the sequence (test cases) which we have to run selected by child, for waveform WAVE=1 so that will verify through simulation.

Figure 1: Compilation

Figure 2: Simulation

- Debug Strategy:

| Debug method | Description |
| --- | --- |
| Simulation log analysis | Analyze simulation logs to identify UVM errors, warnings, and fatal messages |
| Waveform debugging | Analyze AHB signals to identify protocol timing issues |
| Transaction debugging | Check transactions generated by the master and slave agents |
| Scoreboard debugging | Analyze expected and actual transaction mismatches |
| Assertion debugging | Identify protocol violations reported by interface assertions |
| Coverage debugging | Identify uncovered bins and missing test scenarios |
| Random seed debugging | Reproduce failures using the same random seed |
| Driver debugging | Check driver signal generation and handshake behavior |
| Monitor debugging | Verify correct sampling and transaction reconstruction |
| Regression debugging | Analyze failed tests from regression logs |